- RTL code

- module r_m_m #(parameter WIDTH = 4 , parameter PIPELINE_ENABLE = 1 , parameter RSTTYP = "SYNC" ) (

- input [WIDTH-1:0] a,

- input clk , rst , ce ,

- output  [WIDTH-1:0] out

- );

- reg [WIDTH-1 :0]  out_seq ;

- generate

- if ( RSTTYP == "ASYNC")begin

- always @ (posedge clk or posedge rst ) begin

- if (rst)

- out_seq <= {WIDTH{1'b0}};

- else if (ce) begin

- out_seq <= a ;

- end

- end

- end

- else begin

- always @ (posedge clk  ) begin

- if (rst)

- out_seq <= {WIDTH{1'b0}};

- else if (ce) begin

- out_seq <= a ;

- end

- end

- end

- endgenerate

- assign out = ( PIPELINE_ENABLE )?  out_seq : a ;

- endmodule

- module dsp #(

- // Parameters

- parameter A0REG         = 0,

- parameter A1REG         = 1,

- parameter B0REG         = 0,

- parameter B1REG         = 1,

- parameter CREG          = 1,

- parameter DREG          = 1,

- parameter MREG          = 1,

- parameter PREG          = 1,

- parameter CARRYINREG    = 1,

- parameter CARRYOUTREG   = 1,

- parameter OPMODEREG     = 1,

- parameter CARRYINSEL    =  "OPMODE5",

- parameter B_INPUT       = "DIRECT",

- parameter RSTTYPE       = "SYNC"

- )(

- // Input Ports

- input  wire [17:0] A,

- input  wire [17:0] B,

- input  wire [17:0] D,

- input  wire [47:0] C,

- input  wire        CLK,

- input  wire        CARRYIN,

- input  wire  [7:0] opmode_in,

- input  wire [17:0] BCIN,

- input  wire        RSTA,

- input  wire        RSTB,

- input  wire        RSTM,

- input  wire        RSTP,

- input  wire        RSTC,

- input  wire        RSTD,

- input  wire        RSTCARRYIN,

- input  wire        RSTOPMODE,

- input  wire        CEA,

- input  wire        CEB,

- input  wire        CEM,

- input  wire        CEP,

- input  wire        CEC,

- input  wire        CED,

- input  wire        CECARRYIN,

- input  wire        CEOPMODE,

- input  wire [47:0] PCIN,

- // Output Ports

- output wire [17:0] BCOUT,   // assign BCOUT =  B1_OUT ; // CONCAT_D_A_B { D[11:0] , A1_OUT , B1_OUT  }

- output wire [47:0] PCOUT, // assign pcout = p ;

- output wire [47:0] P,

- output reg [35:0] M,

- output wire        CARRYOUT,

- output wire        CARRYOUTF

- );

- // internal signals

- wire [ 17:0 ] D_OUT ;

- wire [ 17:0 ] A0_OUT ;

- wire [ 17:0 ] A1_OUT ;

- wire [ 17:0 ] B0_OUT ;

- wire [ 17:0 ] B1_OUT ;

- wire [ 47:0 ] C_OUT ;

- wire [ 47:0 ] CONCAT_D_A_B ;

- reg [ 17:0 ] B_MUX_BCIN ;

- wire [ 17:0 ] PRE_OUT ;

- wire [ 35:0 ] MULT_OUT ;

- wire [ 35:0 ] M_OUT ;

- reg  OPMAODE_MUX_CARRYIN ;

- reg [ 47:0 ] MUX_X_OUT ;

- reg [ 47:0 ] MUX_Z_OUT ;

- wire [ 47:0 ] POST_OUT ;

- wire [47:0] P_OUT ;

- wire CARRYOUT_OUT ;

- wire CARRYOUT_IN ;

- wire [17:0] PRE_OUT_MUX_B0_OUT ;

- wire CARRYIN_OUT ;

- wire [7:0] opmode ;

- // CONNECTIONS

- assign  CONCAT_D_A_B  = { D[11:0] , A1_OUT , B1_OUT  } ;

- assign BCOUT = B1_OUT ;

- assign CARRYOUT = CARRYOUT_OUT ;

- assign CARRYOUTF = CARRYOUT_OUT ;

- assign PCOUT = P_OUT ;

- assign P = P_OUT ;

- // mux for carryin

- always @(*) begin

- case (CARRYINSEL)

- "OPMODE5" :  OPMAODE_MUX_CARRYIN = opmode [5] ;

- "CARRYIN" : OPMAODE_MUX_CARRYIN = CARRYIN ;

- default : OPMAODE_MUX_CARRYIN = 1'b0 ;

- endcase

- end

- // mux for B

- always @(*) begin

- case (B_INPUT)

- "DIRECT" :  B_MUX_BCIN = B ;

- "CASCADE" : B_MUX_BCIN = BCIN ;

- default : B_MUX_BCIN = 18'b0 ;

- endcase

- end

- // PRE IMP

- assign PRE_OUT = ( opmode [6])?   ( D_OUT - B0_OUT ) : ( D_OUT + B0_OUT ) ;

- // mux for pre or b0_out

- assign  PRE_OUT_MUX_B0_OUT = ( opmode [4])? PRE_OUT : B0_OUT ;

- // mult imp

- assign MULT_OUT = B1_OUT * A1_OUT ;

- // mux x

- always @(*) begin

- case (opmode [1:0])

- 2'b00 : MUX_X_OUT = 48'b0 ;

- 2'b01 : MUX_X_OUT = {12'b0 , M_OUT } ;

- 2'b10 : MUX_X_OUT = P_OUT ;

- 2'b11 : MUX_X_OUT = CONCAT_D_A_B ;

- endcase

- end

- // mux x

- always @(*) begin

- case (opmode [3:2])

- 2'b00 : MUX_Z_OUT = 48'b0 ;

- 2'b01 : MUX_Z_OUT = PCIN ;

- 2'b10 : MUX_Z_OUT = P_OUT ;

- 2'b11 : MUX_Z_OUT = C_OUT ;

- endcase

- end

- // POST IMP

- assign { CARRYOUT_IN , POST_OUT } = (opmode [7] )? (MUX_Z_OUT - (MUX_X_OUT+CARRYIN_OUT)) : (MUX_Z_OUT + MUX_X_OUT );

- // instantiate the r_m_m

- // 5 modules with WIDTH = 18

- r_m_m #(18, A0REG, RSTTYPE) u0 (.a(A),                  .clk(CLK), .rst(RSTA),        .ce(CEA),        .out(A0_OUT));

- r_m_m #(18, A1REG, RSTTYPE) u4 (.a(A0_OUT),             .clk(CLK), .rst(RSTA),        .ce(CEA),        .out(A1_OUT));

- r_m_m #(18, B0REG, RSTTYPE) u3 (.a(B_MUX_BCIN),         .clk(CLK), .rst(RSTB),        .ce(CEB),        .out(B0_OUT));

- r_m_m #(18, B1REG, RSTTYPE) u1 (.a(PRE_OUT_MUX_B0_OUT), .clk(CLK), .rst(RSTB),        .ce(CEB),        .out(B1_OUT));

- r_m_m #(18, DREG, RSTTYPE) u2 (.a(D),                   .clk(CLK), .rst(RSTD),        .ce(CED),        .out(D_OUT));

- // 2 modules with WIDTH = 48

- r_m_m #(48, CREG, RSTTYPE) u5 (.a(C),                   .clk(CLK), .rst(RSTC),        .ce(CEC),        .out(C_OUT));

- r_m_m #(48, PREG, RSTTYPE) u6 (.a(POST_OUT),            .clk(CLK), .rst(RSTP),        .ce(CEP),        .out(P_OUT));

- // 1 module with WIDTH = 36

- r_m_m #(36, MREG, RSTTYPE) u7 (.a(MULT_OUT),            .clk(CLK), .rst(RSTM),        .ce(CEM),        .out(M_OUT));

- // 1 module with WIDTH = 8

- r_m_m #(8, OPMODEREG, RSTTYPE) u8 (.a(opmode_in),       .clk(CLK), .rst(RSTOPMODE),   .ce(CEOPMODE),   .out(opmode));

- // 2 modules with WIDTH = 1

- r_m_m #(1, CARRYINREG, RSTTYPE)  u9  (.a(OPMAODE_MUX_CARRYIN), .clk(CLK), .rst(RSTCARRYIN),  .ce(CECARRYIN),  .out(CARRYIN_OUT));

- r_m_m #(1, CARRYOUTREG, RSTTYPE) u10 (.a(CARRYOUT_IN),         .clk(CLK), .rst(RSTCARRYIN),  .ce(CECARRYIN),  .out(CARRYOUT_OUT));

- // BUFFER FOR M

- always @(posedge CLK ) begin

- M <= M_OUT;

- end

- endmodule

2. Testbench code

module dsp_tb();

reg [17:0] A ;

reg [17:0] B ;

reg [17:0] D ;

reg [47:0] C ;

reg        CLK ;

reg        CARRYIN ;

reg  [7:0] opmode_in ;

reg [17:0] BCIN ;

reg        RSTA ;

reg        RSTB ;

reg        RSTM ;

reg        RSTP ;

reg        RSTC ;

reg        RSTD ;

reg        RSTCARRYIN ;

reg        RSTOPMODE ;

reg        CEA ;

reg        CEB ;

reg        CEM ;

reg        CEP ;

reg        CEC ;

reg        CED ;

reg        CECARRYIN ;

reg        CEOPMODE ;

reg [47:0] PCIN ;

wire [17:0] BCOUT_dut;

wire [47:0] PCOUT_dut;

wire [47:0] P_dut;

wire [35:0] M_dut;

wire        CARRYOUT_dut ;

wire        CARRYOUTF_dut ;

// Clock generation

initial CLK = 0;

always #5 CLK = ~CLK;

// DUT instantiation

dsp dut(

A, B, D, C,

CLK, CARRYIN, opmode_in, BCIN,

RSTA, RSTB, RSTM, RSTP, RSTC, RSTD,

RSTCARRYIN, RSTOPMODE,

CEA, CEB, CEM, CEP, CEC, CED,

CECARRYIN, CEOPMODE,

PCIN,

BCOUT_dut, PCOUT_dut, P_dut, M_dut,

CARRYOUT_dut, CARRYOUTF_dut

);

initial begin

A  = $random;

B  = $random;

D  = $random;

C  = $random;

CARRYIN  = 0;

opmode_in  = $random;

BCIN  = $random;

RSTA  = 1; RSTB  = 1; RSTM  = 1; RSTP  = 1;

RSTC  = 1; RSTD  = 1; RSTCARRYIN  = 1; RSTOPMODE  = 1;

CEA  = 0; CEB  = 0; CEM  = 0; CEP  = 0;

CEC  = 0; CED  = 0; CECARRYIN  = 0; CEOPMODE  = 0;

PCIN  = $random;

@(negedge CLK);

if (BCOUT_dut != 0 || PCOUT_dut != 0 || P_dut != 0 || M_dut != 0 || CARRYOUT_dut != 0 || CARRYOUTF_dut != 0) begin

$display("error in resetting");

$stop;

end

RSTA  = 0; RSTB  = 0; RSTM  = 0; RSTP  = 0;

RSTC  = 0; RSTD  = 0; RSTCARRYIN  = 0; RSTOPMODE  = 0;

CEA  = 1; CEB  = 1; CEM  = 1; CEP  = 1;

CEC  = 1; CED  = 1; CECARRYIN  = 1; CEOPMODE  = 1;

// path1

opmode_in = 8'b11011101;

A = 18'd20; B = 18'd10; C = 48'd350; D = 18'd25;

BCIN = $random; PCIN = $random; CARRYIN = $random;

repeat (4) @(negedge CLK);

if (BCOUT_dut != 18'hf || M_dut != 36'h12c || P_dut != 48'h32 || PCOUT_dut != 48'h32 || CARRYOUT_dut != 0 || CARRYOUTF_dut != 0) begin

$display("error in path1");

$stop;

end

// path2

opmode_in = 8'b00010000;

A = 18'd20; B = 18'd10; C = 48'd350; D = 18'd25;

BCIN = $random; PCIN = $random; CARRYIN = $random;

repeat (4) @(negedge CLK);

if (BCOUT_dut != 18'h23 || M_dut != 36'h2bc || P_dut != 0 || PCOUT_dut != 0 || CARRYOUT_dut != 0 || CARRYOUTF_dut != 0) begin

$display("error in path2");

$stop;

end

// path3

opmode_in = 8'b00001010;

A = 18'd20; B = 18'd10; C = 48'd350; D = 18'd25;

BCIN = $random; PCIN = $random; CARRYIN = $random;

repeat (4) @(negedge CLK);

if (BCOUT_dut != 18'ha || M_dut != 36'hc8 || P_dut != PCOUT_dut || CARRYOUT_dut != CARRYOUTF_dut) begin

$display("error in path3");

$stop;

end

// path4

opmode_in = 8'b10100111;

A = 18'd5; B = 18'd6; C = 48'd350; D = 18'd25;

BCIN = $random; PCIN = 48'd3000; CARRYIN = $random;

repeat (4) @(negedge CLK);

if (BCOUT_dut != 18'h6 || M_dut != 36'h1e || P_dut != 48'hfe6fffec0bb1 || PCOUT_dut != 48'hfe6fffec0bb1 || CARRYOUT_dut != 1 || CARRYOUTF_dut != 1) begin

$display("error in path4");

$stop;

end

$stop;

end

endmodule

3. Do file

vlib work

vlog dsp48A1_tb.v dsp48A1.v

vsim -voptargs=+acc dsp_tb

add wave *

run -all

#quit -sim

4. QuestaSim Snippets

5. Constraint File

## Clock signal

set_property -dict { PACKAGE_PIN W5   IOSTANDARD LVCMOS33 } [get_ports clk]

create_clock -add -name sys_clk_pin -period 10.00 -waveform {0 5} [get_ports clk]

6. Elaboration (“Messages” tab & Schematic snippets)

7. Synthesis (“Messages” tab, Utilization report, timing report & Schematic snippets)

8. Implementation (“Messages” tab, Utiliz

9. Linting (snippets showing no errors)

Qlint dosnt work on my device